DATA COMMUNICATION

AND COMPUTER NETWORKS

Complete Study Notes — Beautified Edition

Covers: Network Fundamentals • OSI Model • Topologies • Transmission Media

Line Coding • Switching Methods • TDM • Exam Traps • Master Memory Sheet

# Table of Contents

# Chapter 1 — Introduction to Computer Networks

## 1.1 What Is a Computer Network?

A computer network is a collection of interconnected computers, devices, and communication equipment that can exchange information and share resources.

The key word is “interconnected.” A network is not merely a collection of computers sitting in the same room — the devices must have a means of communicating (copper cable, optical fiber, radio, microwave, or another medium).

☆ Simple Picture Computer A → Network → Computer B A network allows communication, data exchange, remote services, and resource sharing.

## 1.2 Goals of Computer Networks

| Goal | Explanation |
| --- | --- |
| Resource Sharing | Many computers use a common resource — files, printers, storage, internet, databases, applications, servers. |
| Communication | Email, instant messaging, voice calls, video conferencing, collaboration systems. |
| Data Sharing | A computer can access information stored on another computer or server. |
| Remote Access | A user can access resources from another location. |
| Reliability | Alternate paths and redundant devices keep service running if one component fails. |
| Cost Reduction | Shared resources reduce the amount of hardware required. |
| Centralized Management | A server can centrally provide services, authentication, files, and databases. |
| Scalability | A well-designed network allows new devices/users to be added without a full rebuild. |

## 1.3 Applications of Computer Networks

- Web browsing

- Email

- File sharing

- Online banking

- Online education

- Cloud computing

- Video conferencing

- Remote login

- Online gaming

- Database access

- Printing

- Social networking

- Distributed computing

- Industrial communication

Exam tip: If computers or digital devices communicate, exchange information, or share resources — it is a network application.

# Chapter 2 — Network Structure and Architecture

## 2.1 What Is Network Architecture?

Network architecture describes how computers, clients, servers, and services interact. Two important architectures: Client-Server and Peer-to-Peer.

☆ Don't Confuse ARCHITECTURE = how systems interact TOPOLOGY = how devices are physically/logically connected

## 2.2 Client-Server Architecture

A server provides services; clients request those services. Example: a web browser (client) requests resources from a web server.

| Advantages | Disadvantages |
| --- | --- |
| Centralized administration & security | Server failure may affect many clients |
| Easier backup, centralized data | Server can become a bottleneck |
| Suitable for large organizations | Requires dedicated administration |

## 2.3 Peer-to-Peer Architecture

Computers act as both clients and servers. Example: PC A can request a file from PC B, and PC B can equally request from PC A.

| Advantages | Disadvantages |
| --- | --- |
| Simple for small networks | Difficult to manage as network grows |
| No dedicated central server required | Security becomes harder to enforce |
| Direct resource sharing | Backup and administration less centralized |

## 2.4 Architecture vs Topology

A network can combine Client-Server architecture with a Star topology — no contradiction, since the two terms describe different properties (interaction vs. arrangement).

# Chapter 3 — Network Services

## 3.1–3.2 Service vs Protocol

☆ Golden Rule SERVICE = WHAT functionality is provided PROTOCOL = HOW communication is performed (rules)

A protocol is a set of rules governing communication between entities; a service is the functionality made available through those rules.

## 3.3–3.6 Types of Service

| Service Type | Key Idea | Analogy |
| --- | --- | --- |
| Connection-Oriented | Establish → Transfer → Release | Telephone call |
| Connectionless | Packets sent independently, no setup required | Postal system |
| Reliable | Uses acknowledgements, retransmission, sequencing, error detection/recovery | Registered post |
| Unreliable | No guaranteed delivery; lower overhead, faster | Ordinary postcard |

Note: “Reliable” does not mean physical failure is impossible — it means mechanisms exist to try to achieve reliable delivery.

# Chapter 4 — The OSI Reference Model

## 4.1 Meaning of OSI

OSI = Open Systems Interconnection. It divides communication into seven layers.

☆ Mnemonic (Top → Bottom) APSTNDP “Arey Pagle Suno Toh Nah Diaper Pehen” Application – Presentation – Session – Transport – Network – Data Link – Physical

## 4.2 Why Layers?

Layering divides networking work into manageable responsibilities. Each layer uses services from the layer below and provides services to the layer above.

- Modularity

- Easier troubleshooting

- Easier design

- Easier standardization

- Easier modification

## 4.3–4.9 The Seven Layers

| Layer | Function | Memory Cue |
| --- | --- | --- |
| 7. Application | Network services for applications (web, email, file transfer) | Application = Services |
| 6. Presentation | Translation, formatting, encryption/decryption, compression | Presentation = Representation |
| 5. Session | Establish, manage, and terminate communication sessions | Session = Conversation |
| 4. Transport | End-to-end delivery, segmentation, reassembly, flow control, ports | Transport = End-to-End |
| 3. Network | Logical (IP) addressing, routing, forwarding, packets | Network = Routing |
| 2. Data Link | MAC addressing, framing, local link delivery, medium access | Data Link = MAC / Frame |
| 1. Physical | Electrical/optical/radio signals, voltage, connectors, media | Physical = Bits + Signals |

Data Link asks “which device on this local link?” while Network asks “which network/path should the packet use?”

# Chapter 5 — Encapsulation and Decapsulation

## 5.1–5.2 Encapsulation & PDU Names

As data moves down the layers for transmission, each layer adds its own control information (a header, and sometimes a trailer).

☆ PDU Chain (memorize this!) DATA → SEGMENT → PACKET → FRAME → BITS Transport→Segment ¦ Network→Packet ¦ Data Link→Frame ¦ Physical→Bits

## 5.3–5.4 Decapsulation & Peer-Layer Communication

At the receiver, the reverse process (decapsulation) strips each header/trailer as data moves up the layers: Bits → Frame → Packet → Segment → Data.

Corresponding layers on two computers are said to communicate “logically” (e.g., Transport A ↔ Transport B), even though there is no direct cable between them — actual transmission always goes down through all layers, across the medium, and back up on the receiving side.

# Chapter 6 — Network Topology

## 6.1 What Is Topology?

Topology describes the arrangement or connection pattern of devices in a network. Important types: Bus, Star, Ring, Mesh.

## 6.2–6.5 Topology Types

| Topology | Structure | Advantages | Disadvantages |
| --- | --- | --- | --- |
| Bus | Devices share one main backbone cable | Simple, inexpensive for small networks | Backbone failure affects all; collisions; hard to expand |
| Star | Each device connects to a central switch | Easy install/manage; single cable failure isolates only one device | Central device failure disconnects everyone |
| Ring | Devices form a closed loop | Orderly data flow | Single break can disrupt the loop (unless dual-ring) |
| Mesh | Every device directly connected to every other | High reliability, multiple paths, strong redundancy | Expensive, many cables/ports, poor scalability |

☆ Quick Recall STAR = Central device BUS = Main backbone RING = Closed loop MESH = Many direct connections

## 6.5 Mesh — Link Count Formula

For a full mesh with n devices:

Number of links = n(n − 1) / 2

- Example: n = 5 → links = 5 × 4 / 2 = 10

- Example: n = 20 → links = 20 × 19 / 2 = 190

This demonstrates why a full mesh becomes expensive very quickly as devices increase.

## 6.6 Topology Design Considerations

- Cost

- Reliability

- Scalability

- Ease of maintenance

- Number of devices

- Cabling

- Failure tolerance

- Management

# Chapter 7 — Physical Layer and Transmission Media

## 7.1–7.2 Transmission Media Overview

Transmission media is the path through which a signal travels from sender to receiver. Two broad categories: Guided (wired) and Unguided (wireless).

## 7.3–7.6 Guided Media

| Medium | Description | Key Feature |
| --- | --- | --- |
| Twisted Pair (UTP/STP) | Insulated copper wires twisted together to reduce interference/crosstalk | Copper → electrical signal; UTP cheap & common, STP shielded & costlier |
| Coaxial Cable | Central conductor + insulation + metallic shield + outer jacket | Shield protects against interference |
| Optical Fiber | Transmits data as light through glass/plastic fiber | Very high bandwidth, low attenuation, immune to EMI, long distance |

- Single-mode fiber: one main light path — used for long-distance, high performance.

- Multimode fiber: multiple light paths — used for shorter distances.

## 7.7–7.10 Unguided Media

| Medium | Typical Use |
| --- | --- |
| Radio Waves | Wireless networking, cellular, radio, IoT |
| Microwaves | High-frequency directional links; needs line-of-sight |
| Infrared | Short-range; blocked by obstacles; not for wide-area coverage |

☆ Guided vs Unguided GUIDED = physical path (Twisted Pair, Coaxial, Fiber) UNGUIDED = free space (Radio, Microwave, Infrared)

# Chapter 8 — Line Coding Schemes

## 8.1–8.2 What Is Line Coding?

Line coding determines how digital bits are represented as a digital signal for transmission over a medium. Key properties to compare: synchronization, DC component, bandwidth, number of transitions, error detection, complexity.

## 8.3–8.9 Coding Schemes

| Scheme | Rule | Notes |
| --- | --- | --- |
| Unipolar NRZ | 1 → +V, 0 → 0V | Has DC component; sync difficult on long identical runs |
| Polar NRZ-L | 1 → +V, 0 → −V (level represents bit) | Simple, but long runs hurt sync; DC depends on data |
| NRZ-I | 1 → transition, 0 → no transition | Transition (not level) represents the bit |
| RZ | Signal returns to zero mid-bit | Better sync, but needs more bandwidth |
| Manchester | Transition in the MIDDLE of every bit | Excellent sync/clock recovery; higher bandwidth need |
| Differential Manchester | Middle transition = clock; transition at START = data | Convention-dependent 0/1 assignment |
| AMI | 0 → 0V; 1s alternate +V/−V | Reduced DC component; bipolar violations aid error detection |

☆ Level vs Transition NRZ-L: LEVEL represents the bit NRZ-I: TRANSITION behaviour represents the bit

## 8.10–8.12 DC Component, Synchronization & Bandwidth Trade-off

A DC component appears when the average signal level isn’t centered on zero — undesirable for systems that don’t pass constant DC well.

More transitions generally aid synchronization but demand more bandwidth (higher-frequency content).

| Scheme | Transitions | Sync Quality | Bandwidth |
| --- | --- | --- | --- |
| Simple NRZ | Fewer | Can be difficult on long runs | Efficient (lower) |
| Manchester | More (every bit) | Excellent | Higher requirement |

# Chapter 9 — Switching Methods

## 9.1–9.4 Circuit Switching

Establishes a dedicated communication path before data transfer. Classic example: the traditional telephone system.

☆ Three Stages 1. Connection Establishment 2. Data Transfer 3. Connection Release

Drawback: resources stay reserved even when the sender has nothing to transmit — wasteful for bursty computer traffic.

## 9.5–9.8 Packet Switching

A message is divided into smaller packets that share network resources and may travel via different paths/routers to the destination.

| Advantages | Disadvantages |
| --- | --- |
| Efficient resource sharing | Variable delay |
| Suited to bursty computer traffic | Queuing and congestion possible |
| No continuous path reservation needed | Packet loss / out-of-order arrival |

## 9.9 Circuit vs Packet Switching

| Aspect | Circuit Switching | Packet Switching |
| --- | --- | --- |
| Path | Dedicated | Shared, dynamic |
| Setup | Required before transfer | Not required |
| Best for | Continuous traffic | Bursty computer traffic |
| Analogy | Reserve the road | Share the road |

# Chapter 10 — Time Division Multiplexing (TDM)

## 10.1–10.3 Multiplexing & TDM Basics

Multiplexing allows multiple signals/users to share one channel. A MUX combines input streams; a DEMUX separates them at the receiving end. TDM divides the shared channel into time slots, grouped into frames.

☆ Core Idea MUX = Combine DEMUX = Separate TDM = Division by TIME

## 10.4–10.5 Synchronous vs Statistical TDM

| Type | Slot Assignment | Advantage | Disadvantage |
| --- | --- | --- | --- |
| Synchronous TDM | Fixed slot per input, always reserved | Simple, predictable | Unused slots waste bandwidth |
| Statistical TDM | Slots assigned dynamically based on actual traffic | Better utilization for bursty traffic | More complex control needed |

## 10.6 TDM Calculations

Frame duration = N × T   (N = number of users, T = slot duration)

- Example: N = 4 users, T = 2 ms → Frame duration = 4 × 2 = 8 ms

Data rate = slots/frame × bits/slot × frames/second

- Example: 8 slots/frame × 1 bit/slot × 1000 frames/sec = 8000 bits/second

## 10.7 TDM vs FDM

☆ Division Basis TDM → divides by TIME FDM → divides by FREQUENCY

# Chapter 11 — Important Comparisons

## 11.1 Data Link vs Network

| Data Link | Network |
| --- | --- |
| MAC address, Frame, local/link delivery | IP address, Packet, routing, forwarding |

## 11.2 Network vs Transport

| Network | Transport |
| --- | --- |
| Routing, IP addressing, packet forwarding | End-to-end delivery, segmentation, reassembly, reliability, flow control, ports |

## 11.3 Application vs Presentation vs Session

| Layer | Focus |
| --- | --- |
| Application | Network services for applications |
| Presentation | Representation, formatting, encryption, compression |
| Session | Establishment, management, termination of sessions |

## 11.4–11.5 Media Comparisons

| Medium | Material | Signal | Notes |
| --- | --- | --- | --- |
| Twisted Pair | Copper | Electrical | UTP / STP |
| Coaxial | Copper | Electrical | Central conductor + shield |
| Fiber | Glass/plastic | Light | Very high bandwidth, long distance, EMI-immune |

## 11.6 Circuit vs Packet (Recap)

| Circuit Switching | Packet Switching |
| --- | --- |
| Dedicated path, reserved resources, connection setup, good for continuous traffic | Packets, shared resources, dynamic forwarding, good for bursty traffic |

## 11.7 Star vs Bus vs Ring vs Mesh (Recap)

| Star | Bus | Ring | Mesh |
| --- | --- | --- | --- |
| Central device | Main backbone | Closed loop | Multiple direct links |

## 11.8–11.9 NRZ vs Manchester vs Differential Manchester

| Scheme | Sync Source | Data Representation |
| --- | --- | --- |
| NRZ | Level (weak on long runs) | Direct level per bit |
| Manchester | Middle transition (every bit) | Direction of middle transition |
| Differential Manchester | Middle transition (every bit) | Transition (or not) at start of bit |

# Chapter 12 — Common Examination Traps

Q: Which OSI layer performs routing?

A: Network layer (not Data Link) — routing is a Layer-3 function.

Q: Which OSI layer uses MAC addresses?

A: Data Link layer.

Q: Which layer provides reliable end-to-end delivery?

A: Transport layer.

Q: Which layer handles encryption, compression, translation?

A: Presentation layer.

Q: Which layer manages communication sessions?

A: Session layer.

Q: Difference between service and protocol?

A: Service = WHAT is provided; Protocol = HOW it's carried out.

Q: Which topology uses a central switch?

A: Star topology.

Q: Which topology uses a backbone?

A: Bus topology.

Q: Which topology connects every device to every other?

A: Full mesh — links = n(n−1)/2.

Q: Which medium uses light?

A: Optical fiber.

Q: Which medium is immune to electromagnetic interference?

A: Optical fiber.

Q: Key feature of Manchester encoding?

A: A transition in the middle of every bit period.

Q: Why does Manchester need more bandwidth?

A: It contains more signal transitions.

Q: What does circuit switching establish?

A: A dedicated path.

Q: What does packet switching do?

A: Divides data into packets and shares network resources.

Q: What does TDM divide?

A: The channel by time (into time slots).

Q: Synchronous TDM — source has no data?

A: Its fixed slot remains unused (wasted).

# Chapter 13 — One Complete End-to-End Example

Scenario: a browser requests a resource from a remote server. Follow the data down the stack, across the network, and back up at the server.

| Step | Layer | What Happens |
| --- | --- | --- |
| 1 | Application | The Application layer provides network services to the browser. |
| 2 | Presentation | Data formatting, encryption, or compression may be applied. |
| 3 | Session | The communication session is managed. |
| 4 | Transport | Data is segmented for end-to-end delivery: DATA → SEGMENT. |
| 5 | Network | Logical addressing and routing added: SEGMENT → PACKET. |
| 6 | Data Link | Packet placed into a frame with MAC addressing: PACKET → FRAME. |
| 7 | Physical | Frame becomes bits as physical signals: FRAME → 0101…, sent over the medium. |

A simplified path: Computer → Router → Router → Server. At the destination, the process reverses (decapsulation): Bits → Frame → Packet → Segment → Data → Application.

# Chapter 14 — Master Memory Sheet

## OSI Order

☆ APSTNDP “Arey Pagle Suno Toh Nah Diaper Pehen” A-Application, P-Presentation, S-Session, T-Transport, N-Network, D-Data Link, P-Physical

## OSI Function Memory

| Layer | Memory Cue |
| --- | --- |
| Application | Network services |
| Presentation | Format / encryption / compression |
| Session | Manage communication sessions |
| Transport | End-to-end delivery |
| Network | IP / routing / forwarding |
| Data Link | MAC / frames / local link |
| Physical | Bits / signals / medium |

## PDU, Address & Topology Memory

☆ PDU Chain DATA → SEGMENT → PACKET → FRAME → BITS Transport→Segment ¦ Network→Packet ¦ Data Link→Frame ¦ Physical→Bits

☆ Addressing Port → Transport IP → Network MAC → Data Link

☆ Topology Star = Central device Bus = Backbone Ring = Loop Mesh = Many direct connections

## Media, Line Coding, Switching & TDM Memory

| Category | Memory Cue |
| --- | --- |
| Media | Twisted Pair=Copper ¦ Coaxial=Conductor+Shield ¦ Fiber=Light ¦ Radio/Microwave/Infrared=Wireless |
| Line Coding | NRZ-L=Level ¦ NRZ-I=Transition ¦ RZ=Returns to zero ¦ Manchester=Middle transition ¦ AMI=1s alternate polarity |
| Switching | Circuit=Dedicated path ¦ Packet=Packets + shared resources |
| TDM | MUX=Combine ¦ DEMUX=Separate ¦ Synchronous=Fixed slots ¦ Statistical=Dynamic slots |

# Chapter 15 — Last-Minute Examination Revision

If the exam is about to begin and only five minutes remain, remember the following:

- A computer network allows interconnected devices to communicate and share resources.

- Client-server: Server provides services. Client requests services.

- Peer-to-peer: Peers can act as both clients and servers.

- Star = Central device. Bus = Backbone. Ring = Closed loop. Mesh = Multiple direct connections.

- OSI: APSTNDP — Arey Pagle Suno Toh Nah Diaper Pehen.

- OSI functions: Application→Services, Presentation→Representation, Session→Sessions, Transport→End-to-end, Network→Routing/IP, Data Link→MAC/Frames, Physical→Bits/Signals.

- PDU sequence: DATA → SEGMENT → PACKET → FRAME → BITS.

- Addressing: Port→Transport, IP→Network, MAC→Data Link.

- Transmission media — Guided: Twisted Pair, Coaxial, Fiber. Unguided: Radio, Microwave, Infrared.

- Fiber uses light, offers very high bandwidth, and resists electromagnetic interference.

- Manchester: middle transition, excellent synchronization, higher bandwidth requirement.

- AMI: 0 = zero level; 1s = alternating polarity.

- Circuit switching = dedicated path. Packet switching = packets share network resources.

- TDM divides by time. Synchronous = fixed slots. Statistical = dynamic slots by traffic.

- Service = WHAT is provided. Protocol = HOW communication is performed.

- Most important distinction: MAC→Data Link, IP→Network, Port→Transport.

- Bits must be represented as physical signals for transmission.

- Circuit switching reserves a path; packet switching shares network resources.

## Final Professor’s Remark

Do not merely memorize answers to multiple-choice questions — remember the reasoning chain behind each fact:

☆ Reasoning Chains “Which layer uses MAC addresses?” → MAC → Data Link → Frame “Which layer performs routing?” → IP → Network → Routing → Packet “Why is Manchester useful?” → Middle transition → Clock sync → Easier receiver timing “Why is packet switching efficient?” → Bursty traffic → No permanent reservation → Shared by many → Better utilization “Why is mesh reliable?” → Multiple links → Multiple paths → One failure doesn’t isolate the network

The purpose of studying Data Communication and Computer Networks is not to memorize isolated definitions — it is to understand how a message travels from one application to another: how it is represented, transported, routed, framed, converted to physical signals, and how the network shares its limited communication resources. Once that complete picture is understood, the individual multiple-choice questions become much easier.